Pressure Testing (mini-bulk) Portable Refillable Containers (PRC)

- Some, but not all, PRC’s will already have a sticker indicating the last time the tank was pressure tested

- You must document all pressure testing and visual inspections

- Hit the PRC with a baseball bat to ensure the integrity of the tank.  If the tank is not damaged after the “baseball bat test”, proceed to the next step

- Pressure testing must be done every 30 months (2.5 years)

- Prior to pressurizing the PRC, make sure all openings such as vents, caps, valves, etc….. are tight

- Pressurize the PRC to 3 PSI.  CAUTION 5-6 PSI will cause the tank to bulge

- Double check all openings to make sure they are tight

- Spray the PRC with soapy water and look for bubbles

- Maintain 3 PSI for approximately 5 minutes

- During the 5 minute pressure test, conduct a walk around inspection of the PRC and check for spider web cracking, discoloration or weak spots due to exposure from the sun

- Once testing and inspection is complete, place a sticker on the tank

indicating the month and year of the inspection

- Make sure to document all testing and inspection of the PRC

- A visual inspection must be conducted and documented each time a

farmer brings the tank back for refilling